Annex A

Breakdown of Household Energy Consumption

Across All Housing Types

Source: Household Energy Consumption Study 2017

Annex B

MEPS for Incandescent, CFLi & LED bulbs

| Table 1: Current MEPS for Incandescent, CFLi & LED bulbs |  |  |
| --- | --- | --- |
| Type of Lamps* | Wattage (W) | MEPS Cut-off Levels |
| Incandescent | 25 - 200 | Pmax = 0.8 x (0.88√ + 0.049) |
| Compact fluorescent lamps with integrated ballast (CFLi) | ≤ 60 | Pmax = 0.24√ + 0.0103 |
| LED Bulbs | ≤ 60 |  |
| Covered CFLi | ≤ 60 | Pmax¦covered CFLi = Pmax¦bare CFLi /0.95 |

* Lamps with Edison screw and bayonet lamp caps, which are designed to be connected directly to the 230V A.C. mains by means of a socket or connector

Pmax is maximum allowable power consumption

is light output in lumen

| Table 2: Life Cycle Costs of Different Types of Lamps |  |  |  |
| --- | --- | --- | --- |
| Type of Lamp | Inefficient Lamp | Energy Efficient Replacement Lamps |  |
|  | Incandescent (Tungsten-halogen) | CFLi | LED bulb |
| Annual Cost | $11.10 | $3.60 | $3.00 |

Annex B

| Table 3: Proposed Revised MEPS Levels for Incandescent, CFLi & LED bulbs |  |  |
| --- | --- | --- |
| Type of Lamps* | Wattage (W) | MEPS Cut-off Levels |
| Incandescent | 25 - 200 | Pmax = 0.24√ + 0.0103 |
| LED Bulbs | ≤ 60 |  |
| CFLi | ≤ 60 |  |
| Covered CFLi | ≤ 60 | Pmax¦covered CFLi = Pmax¦bare CFLi /0.95 |

* Lamps with Edison screw and bayonet lamp caps, which are designed to be connected directly to the 230V A.C. mains by means of a socket or connector

Pmax is maximum allowable power consumption

is light output in lumen

Annex C

Diagram 1: Energy label for lamps (to be affixed or printed on each lamp packaging)

| Table 4: Proposed energy rating system for CFLni (G24d lamp cap) and T8 LFL of length 0.5 – 1.5m, and their LED direct replacements |  |
| --- | --- |
| Tick rating | Rated Lamp Efficacy, = /P (lm/W) |
| 3-tick | ≥ 135 |
| 2-tick | 110 ≤ < 135 |
| 1-tick | where P is the rated lamp power - is the rated luminous flux in lumen < 110 |

| Lamp Types | CFLni [Lamp cap: G24d] | LFL [Dia. 1-inch (T8 lamp)] 0.5 – 1.5m | LED direct replacement |
| --- | --- | --- | --- |
| Lamp survival | 90% at 2,000h |  | 95% at 1,000h (Measured) 90% at 6,000h (Declared) |
| Lumen maintenance | 80% at 2,000h |  | 85% at 1,000h (Measured) 80% at 6,000h (Declared) |

Table 5: Lamp survival and Lumen maintenance requirements

Annex D

Energy Efficiency of Fluorescent Lamp Ballasts

| Table 6: Energy Efficiency Index (EEI) classification system |  |
| --- | --- |
| EEI Class | Description |
| A1 | Dimmable electronic ballasts |
| A2 | Electronic ballasts with reduced losses |
| A3 | Electronic ballasts |
| B1 | Magnetic ballasts with very low losses |
| B2 | Magnetic ballasts with low losses |
| C | Magnetic ballasts with moderate losses |
| D | Magnetic ballasts with very high losses |

| Table 7: Life Cycle Costs of Different Classes of Ballast |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type of Ballast | Moderate Loss Replacement Ballasts |  | Low Loss Replacement Ballasts |  |
|  | Class B2 Magnetic | Class B1 Magnetic | Class A3 Electronic | Class A2 Electronic |
| Life Cycle Cost (LCC) | $18 | $16 | $9.90 | $15.50 |

Annex E

Fluorescent Lamp Ballasts Not Covered Under Proposed MEPS

Fluorescent lamp ballasts that are not covered under MEPS are listed below:

- Ballasts integrated in lamps (e.g. for HID lamp),

- Ballasts used primarily in D.C. circuits,

- Ballasts not used primarily for general lighting purposes (e.g. ballasts for exit signs, laboratory, ultraviolet, industrial process applications), and

- Ballasts for luminaires that are mounted permanently in furniture which cannot be tested independently from luminaires

Annex F

Examples of Acceptable / Non-Acceptable Display of Energy Efficiency Information in Publicity Materials

| Acceptable Examples |  |
| --- | --- |
| Energy Label displayed such that tick-rating is legible | No. of ticks in font same size / larger than main text |

Annex F

| X Unacceptable Examples |  |
| --- | --- |
| Energy Label with illegible / obstructed no. of ticks | No. of ticks in font smaller than that of other specifications |

Annex G

Applicable Test Standards

Table 8

| Lamp Types | CFLni [Lamp cap: G24d] | LFL [Dia. 1-inch (T8 lamp)] 0.5 – 1.5m |  | LED direct replacement |
| --- | --- | --- | --- | --- |
| Lumen Measurement | CIE 84 |  |  | CIE S 025 |
| Other Performance standards | IEC 60901 |  | IEC 60081 | IEC 62612* |

*Only relevant clauses of IEC 62612 apply.

Upfront and energy costs are amortized to 1,000 hours, which is equivalent to around 1 year of usage, assuming 3 hours of daily usage. Energy costs are estimated based on average 2017 household electricity tariffs.

The ballasts are used with a 36W T8 tube.

Based on 10,000hr of operation which is approximately equivalent to 10 years of operation, assuming 3hr of daily usage, and 2017 average tariff including 7% GST.